# STUDENT NAME: MONIKA P                                                                                  REG NO: 192425059

# CO1 ASSESSMENT 2

# ITA0329 - MOBILE COMPUTING

Q26. A paging scheme sweeps through 4 cells at a time across a 200-cell network. In the worst case, how many paging rounds are needed to locate a subscriber?

Answer: Paging rounds = Total cells ÷ Cells per round

= 200 ÷ 4

= 50. 50 paging rounds.

Q27. A core network is engineered for 100,000 simultaneous connections. During a busy period it receives 40,000 call attempts per minute, each lasting 3 minutes on average. What fraction of the offered traffic ends up blocked?

Answer: Offered load = 40,000 × 3 = 120,000 simultaneous calls.

Blocked fraction = (120,000 − 100,000) ÷ 120,000 × 100

= 16.67%.

16.67%, assuming capacity is the maximum active-call count and excess offered load is blocked.

Q28. A network policy lets 15% of its 600,000 subscribers roam onto partner networks each hour. At any given moment, how many subscribers are likely roaming?

Answer: Roaming subscribers = 15% × 600,000

= 90,000. 90,000 subscribers.

Q29. A switching centre is capable of processing 12,000 calls per second, but is currently handling a peak load of 9,000 calls per second. What percentage of its capacity is being utilized?

Answer: Utilization = (9,000 ÷ 12,000) × 100

= 75%. 75%.

Q30. An operator reuses frequencies with a factor of 4 across a network holding 48 MHz of spectrum, with 200 kHz per channel. How many channels land in each cell?

Answer: Total channels = 48,000 kHz ÷ 200 kHz

= 240. Channels per cell

= 240 ÷ 4

= 60. 60 channels per cell.

Q31. A 900 MHz GSM deployment uses 200 kHz channel spacing across a 30 MHz allocation. How many channels can this spectrum support?

Answer: Number of channels = 30,000 kHz ÷ 200 kHz

= 150. 150 channels.

Q32. Over the course of an hour, a network logs 2500 handoff attempts, 99% of which succeed. How many calls were successfully handed off?

Answer: Successful handoffs = 2,500 × 0.99

= 2,475. 2,475 successful handoffs.

Q33. A mobile handset radiates 12 W, and the link experiences 45 dB of path loss before reaching the base station. What power level does the base station receiver detect?

Answer: Transmit power = 10 log₁₀(12,000 mW)

= 40.79 dBm. Received power

= 40.79 − 45

= −4.21 dBm. −4.21 dBm.

Q34. A 7-cell cluster is deployed where each cell can serve up to 900 simultaneous users. What is the cluster's overall user capacity?

Answer: Cluster capacity = 7 × 900

= 6,300. 6,300 simultaneous users.

Q35. A TDMA carrier's frame spans 4.8 ms and is split into 8 equal slots. How long is each individual slot?

Answer: Slot duration = 4.8 ms ÷ 8

= 0.6 ms. 0.6 ms.

Q36. A mobile device moves at 150 km/h while connected to a 1.8 GHz carrier. What Doppler shift should the receiver expect?

Answer: Speed = 150 ÷ 3.6

= 41.67 m/s. Maximum Doppler shift

= vf/c

= (41.67 × 1.8 × 10⁹) ÷ (3 × 10⁸)

= 250 Hz. 250 Hz (maximum magnitude; actual shift depends on direction of motion).

Q37. A core switch is dimensioned for 250,000 simultaneous calls. It is fielding calls at a rate of 70,000 per minute, each averaging 4 minutes in duration. What share of the offered load gets blocked?

Answer: Offered load = 70,000 × 4

= 280,000 simultaneous calls.

Blocked share = (280,000 − 250,000) ÷ 280,000 × 100

= 10.71%. 10.71%, assuming excess offered load is blocked.

Q38. A hexagonal layout groups 9 cells into a cluster, and each cell can host 600 users. What is the cluster's combined user capacity?

Answer: Cluster capacity = 9 × 600

= 5,400. 5,400 users.

Q39. A switching centre processes 60,000 calls per second, with 0.4% of them dropped due to network issues. How many calls are being dropped every second?

Answer: Dropped calls per second = 60,000 × 0.004

= 240. 240 calls per second.

Q40. A base station in an 1800 MHz network transmits at 43 dBm. At the edge of its 2 km-radius cell, the received signal measures -87 dBm. What path loss has the signal undergone?

Answer: Path loss = Transmit power − Received power

= 43 − (−87)

= 130 dB. 130 dB.

Q41. A single base station covers 2.5 km² of territory, and the operator plans to deploy 150 identical stations. What total geographic area will the rollout cover?

Answer: Total area = 2.5 × 150

= 375 km². 375 km².

Q42. An operator applies a frequency reuse factor of 6 over a 48 MHz spectrum allocation, with each channel taking up 200 kHz. How many channels does each cell get?

Answer: Total channels = 48,000 kHz ÷ 200 kHz

= 240. Channels per cell

= 240 ÷ 6

= 40. 40 channels per cell.

Q43. A paging system processes 6000 users per cycle, and the subscriber base totals 150,000 users. How many paging cycles are needed to sweep the entire base?

Answer: Paging cycles = 150,000 ÷ 6,000

= 25. 25 cycles.

Q44. A phone call runs for 9 minutes, and during that time the network hands the call off roughly every 3 minutes. How many handoffs took place over the call?

Answer: Estimated handoffs = 9 ÷ 3

= 3. Approximately 3 handoffs.

Q45. A receiver picks up a wanted signal at -85 dBm against a noise floor of -115 dBm. What signal-to-noise ratio, in dB, does this represent?

Answer: SNR = Signal power − Noise power

= −85 − (−115)

= 30 dB. 30 dB.

Q46. A 4G cell is serving 250 users simultaneously, each needing a guaranteed 150 kbps. What is the minimum bandwidth the cell must provide to support them all?

Answer: Required aggregate data rate = 250 × 150 kbps

= 37,500 kbps

= 37.5 Mbps. 37.5 Mbps aggregate throughput.

The actual radio bandwidth depends on spectral efficiency.

Q47. A base station handles 1500 subscribers, each averaging 4 calls per hour with a 2.5-minute call length. What total traffic, in Erlangs, does this generate?

Answer: Traffic = (Subscribers × Calls per hour × Average call duration in minutes) ÷ 60.

A = (1,500 × 4 × 2.5) ÷ 60

= 250 Erlangs. 250 Erlangs.

Q48. A network reserves 250 separate control channels, each 12 kHz wide, for signaling. What total bandwidth is tied up in control channels?

Answer: Bandwidth = 250 × 12 kHz

= 3,000 kHz

= 3 MHz. 3 MHz.

Q49. A network is carrying 1500 simultaneous connections, each streaming at 600 kbps on average. What total throughput must the network support?

Answer: Total throughput = 1,500 × 600 kbps

= 900,000 kbps

= 900 Mbps. 900 Mbps.

Q50. A telecom operator wants each tower to cover a 4 km radius (treating coverage as a circle, area = πR²) and needs to blanket a 350 km² service region. Roughly how many towers will the rollout require?

Answer: Area per tower = πR² = π × 4²

= 16π ≈ 50.27 km².

Towers = 350 ÷ 50.27

≈ 6.96; round up to 7.

Approximately 7 towers, under the ideal circular-coverage assumption.